************************************************************************

A DESCRIPTION OF THE TRAINING SET

for learning on a Finite Element Mesh Design problem

************************************************************************

14-April-1994

About the training set

----------------------

This is a complete set of the training examples! Ten different FE mesh

models have been used as a source of examples.

Since it is impossible to take in consideration all different structures

that can appear in praxis the FE models described in the training set

represent a family with the following common features:

* all structures are cylindrical,

* loads are only due to forces or pressures (there is no

thermal influences, ...),

* highly local mesh refinement is not required.

Thus only the rules suitable for this family of structures can

be expected as a result of machine learning with that kind of

training set.

A detailed review of the first five FE models chosen as sources of

training examples can be found in [1]. Details about the other five models

can be obtained on request. The FE meshes for all structures were

'hand-constructed' and modified several times until the numerical results

were accurate enough at acceptable computer time consumption.

FE mesh models were divided into a collection of edges. The edges

were labeled with a combination of a letter and a number. The letter

denotes a mesh model while the consecutive numbers denote individual

edges in the model.

Since edges with more than 12 finite elements are quite rare we decided to

learn the rules only for classes from 1 to 12.

Positive examples

-----------------

The target predicate is a "mesh(E,N)", where E is a name of the edge

and N is the number of finite elements on that edge. For each edge

in the training set the number of finite elements is given. Some

deviations were allowed. So for N=8 or more, for each example mesh(E,N),

additional examples mesh(E,N1), N1=N+-1 were added.

Negative examples

-----------------

Basically the negative examples were constructed by the close-world

assumption which means that they are presented by the combinations

of the names of the edges with all the numbers between 1 and 12

other than those found in the positive examples. Again some

deviations from this were considered. For all the edges with 5 to 7

finite elements deviations by 1 element were not stated as the

negative examples. For the edges with 8 or more elements deviations

by up to 2 elements were tolerated.

In the next step a single class learning will be carried out. It is

decided in this case to use positive examples for all other classes

rather the actual class as negative examples. The permisable

deviations should also be considered.

Background knowledge

--------------------

Background knowledge contains definitions of the predicates that can

be used in the hypotheses about the target relation mesh/2.

Background predicates have the edges as arguments. Background

knowledge can be divided into two parts:

* attribute description of the edges,

* geometric relations between the edges.

The following attributes of the edges which pertain to the FE mesh

are described as the background facts:

* the type of the edge using predicates

long, usual, short, circuit, half_circuit, quarter_circuit,

short_for_hole, long_for_hole, circuit_hole,

half_circuit_hole, quarter_circuit_hole, not_important,

* the supports using predicates

free, one_side_fixed, two_side_fixed, fixed,

* the loads using predicates

not_loaded, one_side_loaded, two_side_loaded, cont_loaded.

The geometric representation is needed because of the relationships

between the edges, which influence to the FE mesh. We believe there

is a connection between the edges which are neighbours or opposite

(parallel) to each other which has a consequence on the appropriate

number of finite elements on those edges.

Both two predicates have two arguments. Each argument (an edge) can

be input or output. For example consider the predicate

neighbour(A,B). For given A, B can be determined; for given B, A can

be determined.

Summary

-------

The examples and background knowledge for learning the FE mesh

design comprised:

*  642 positive examples,

* 3804 background facts (nondeterminate version).

There are two separate versions of background facts, one for

determinate version of GOLEM and the another for nondeterminate

learning algorithm. If possible, nondeterminate learning algorithm

should be used.

The list of all data files with a short description of contents is

given in the file "FILENAMES.DOC".

References

----------

[1]  B. Dolsak. Defining a Geometric Mesh Model for FEM Analyses

Using Artificial Intelligence Techniques (in Slovene, with

English abstract), M.Sc. Thesis, University of Maribor, Faculty

of Technical Sciences, Slovenia, 1991.

[2]  B. Dolsak and S.Muggleton. The Application of Inductive Logic

Programming to Finite Element Mesh Design, Inductive Logic

Programming, Academic Press, 1992, pp. 453-472.

[3]  S. Dzeroski & B. Dolsak. Comparison of ILP Systems on the

Problem of Finite Element Mesh Design, ISSEK Workshop, Bled,

Slovenia, 1992.

[4]  B. Dolsak & A. Jezernik & I. Bratko.  A Knowledge Base for

Finite Element Mesh Design, in review process to be published

in Artificial Intelligence in Engineering, Elsevier Science

Publishers.

COPYRIGHT

---------

The presented data can be accessed without any legal obligations.

In publications please acknowledge Bojan Dolsak from Mechanical

Engineering Department of the Faculty of Technical Sciences in

Maribor, Slovenia as the source of data.

ADDRESS FOR ADDITIONAL INFORMATION

-----------------------------------------------------------------

Bojan DOLSAK, MSc, Mech. Eng.

University of Maribor

Faculty of Technical Sciences

Mechanical Engineering Department	    Tel: +386 62 20 091

Smetanova 17, 62000 Maribor		    Fax: +386 62 22 50 13

SLOVENIA			         E-mail: dolsak@uni-mb.si

-----------------------------------------------------------------